# ThingsBoard SIP Alert Calling - Cloud Test Guide

Engineering test plan and implementation starter for cloud-only alarm-to-phone validation

Document status: Test guide  |  Audience: Engineering team  |  Updated: October 2026

## Decision summary

Start with two free SIP endpoints to prove the ThingsBoard → REST → headless SIP call chain. Add a trial PSTN provider only after the functional path is stable. Verify current provider pricing, trial terms, geographic availability, identity requirements, and emergency-calling rules before any purchase or production use.

# 1. Purpose & scope

This guide validates an entirely cloud-hosted alarm calling path:

ThingsBoard rule chain → REST call → headless soft SIP phone service → cloud SIP provider/PBX → target phone

ThingsBoard does not speak SIP itself. It evaluates telemetry and alarms, then calls an HTTP endpoint. The small service on a cloud VM owns SIP registration, dialing, audio playback, retries/timeouts, and provider-specific behavior. No office PBX is required; all components in this test are cloud-based except the receiving mobile phone.

In scope: functional alarm delivery, SIP registration/re-registration, softphone audio, PSTN trial validation, failure handling, observability, and basic security. Out of scope: emergency calling, regulatory compliance, high-availability production design, and a final provider contract.

# 2. Architecture

┌──────────────────────┐       REST/HTTP        ┌─────────────────────────┐

│ ThingsBoard telemetry │ ─────────────────────> │ FastAPI alert endpoint  │

│ alarms / rule chain   │                        │ API key + validation    │

└──────────────────────┘                        └───────────┬─────────────┘

│

│ ctrl_tcp / subprocess

▼

┌─────────────────────────┐

│ Baresip or PJSUA2       │

│ headless SIP user agent │

└───────────┬─────────────┘

│ SIP + RTP

▼

┌─────────────────────────┐

│ Cloud SIP provider/PBX  │

│ Linphone/Telnyx/Twilio  │

└───────────┬─────────────┘

│ PSTN or SIP

▼

┌─────────────────────────┐

│ Target mobile / softphone│

└─────────────────────────┘

| Component | Responsibility |
| --- | --- |
| ThingsBoard | Telemetry, alarm state, rule-chain branching, REST request and audit trail. |
| Cloud VM + Docker | Runs the protected FastAPI wrapper and headless SIP client; use a small VM during testing. |
| Baresip / PJSUA2 | Registers to SIP, originates calls, handles RTP and audio. |
| Cloud SIP provider/PBX | Routes SIP-to-SIP or SIP-to-PSTN; identity, trial limits, billing, regional rules. |
| Target endpoint | Linphone/Zoiper softphone for Phase 1, or a verified mobile number for Phase 2. |

# 3. Cloud SIP provider options

The entries below are test-path options, not a price quote. Pricing and trial terms change. Verify the provider’s current pricing, limits, required verification, PSTN coverage, taxes, and acceptable-use policy before provisioning.

| Provider / option | Cost model | PSTN? | Best for | Notes |
| --- | --- | --- | --- | --- |
| Linphone free SIP account sip.linphone.org | Free account | No | Phase 1 SIP-to-SIP | Good for two Linphone endpoints; no PSTN termination. |
| sip2sip.info | Free account | Generally no | Basic SIP interoperability | Confirm current availability and service limits. |
| Zadarma | Free tier / usage-based | Yes, subject to account/rules | Low-cost PSTN experiments | Verify current free tier, identity, regions, and rates. |
| Telnyx | Trial credit then usage / numbers | Yes | Developer-friendly PSTN trial | Use a SIP credential connection; trial credit and verification are subject to current terms. |
| Twilio | Trial credit then usage / numbers | Yes | Fast programmable PSTN test | SIP Domain + credential list; trial calls are restricted to verified numbers. |
| VoIP.ms | Small deposit plus usage / DID | Yes | Traditional SIP/PSTN testing | Real DID and broad SIP controls; verify current deposit and rates. |
| Plivo / Vonage / SignalWire | Trial or usage-based | Yes | Alternative programmable voice | Compare SIP support, verification, and regional coverage. |
| 3CX / Grandstream cloud | Hosted PBX subscription / trial | Via trunk | Team PBX-style test | Useful when extension logic and PBX features are required. |

# 4. Recommended test path

Phase 1 — free Linphone accounts: prove registration, dialing, alert body, and softphone-to-softphone audio without PSTN cost.

Phase 2 — Telnyx or Twilio trial: place a real mobile call; verify your own number and use only current trial allowance.

Phase 3 — production selection: choose provider based on geography, caller ID, reliability, monitoring, support, compliance, and verified current pricing.

Do not buy a DID or commit to a paid plan before Phase 1 is green and the PSTN requirements are understood.

# 5. Prerequisites

A small cloud VM (approximately $5/month class, Docker-capable) for the softphone service; actual current price varies by vendor and region.

ThingsBoard Cloud or an existing ThingsBoard instance with permission to edit a rule chain and send telemetry.

A mobile phone with Linphone or Zoiper for SIP-to-SIP testing; a verified mobile number for PSTN trials.

Outbound internet from the VM, DNS resolution, NTP/time synchronization, and firewall control.

A secure secret store or environment variables for SIP password and API key; never put secrets in source control.

# 6. Provider setup

## 6.1 Linphone free SIP account

Create two SIP identities through the current Linphone account flow; record the SIP username and password for each endpoint.

For the headless VM, use one identity. Install Linphone on the mobile and use the second identity.

Use the current registrar shown by Linphone; for this guide the commonly used registrar is sip.linphone.org. Confirm in the account documentation before testing.

Register both endpoints, then dial the mobile identity from the VM. Keep this phase SIP-to-SIP; it does not provide PSTN.

## 6.2 Telnyx — SIP credential connection

Create/verify the account and review current trial credit, identity, geographic, and billing requirements.

Create a SIP Connection using credential-based authentication. Create a SIP username/password; do not use API keys as SIP passwords.

Configure the SIP domain/host and outbound voice profile or equivalent routing to the destination.

If a phone number is required, provision/verify it only after Phase 1; set caller ID according to current provider rules.

Use the provider’s current SIP server, transport, codec, and registration instructions in the final config.

## 6.3 Twilio — SIP Domain and credential list

Create a SIP Domain in the current Twilio Console and select credential-list authentication.

Create a Credential List and add a username/password for the headless softphone. Associate the list with the SIP Domain.

Configure a Voice/termination path and caller ID according to the current Twilio console workflow.

On a trial account, call only verified destination numbers. Confirm the trial announcement/limitations and current credit before testing.

Use the SIP Domain host and transport published by Twilio; do not infer a production endpoint from this example.

## 6.4 Registration values worksheet

| Field | Linphone / SIP-to-SIP | Telnyx / Twilio / other |
| --- | --- | --- |
| Registrar | sip.linphone.org (verify current) | Provider-published SIP server/domain |
| Username | SIP identity username | SIP credential username |
| Password | SIP account password | SIP credential password |
| Transport | UDP for first test; TLS if supported | UDP/TLS per provider; prefer TLS where supported |
| Target | Second Linphone identity | Verified mobile number or SIP URI |

Store actual values in a password manager or environment variables, not in this document.

# 7. Headless softphone service

Recommend Baresip for a small, scriptable headless client or PJSUA2 when a richer programmable media stack is needed. Put a thin REST wrapper in front of it. The wrapper should validate input, authenticate callers, serialize or queue calls, apply a timeout, and log a correlation ID without logging secrets.

## 7.1 Baresip account configuration

; ~/.baresip/accounts

sip:USERNAME@sip.provider.com;transport=tls>;auth_pass=PASSWORD;regint=300

The exact account-line syntax can vary by Baresip build and provider. Preserve the provider’s required URI/transport syntax and test it with the installed version. On a headless server, configure a null audio driver or an audio file driver (for example, null/aufile) so the process does not require a physical sound device. Use a prerecorded WAV or provider-approved media path for deterministic tests.

## 7.2 Minimal FastAPI starter wrapper

Starter code — adapt and test before use. It demonstrates an API-key-protected POST endpoint and uses Baresip’s ctrl_tcp interface or a subprocess fallback. It does not implement a production queue, authorization system, retry policy, TTS, or durable audit store.

# app.py — starter code to adapt/test

import os, shlex, socket, subprocess

from fastapi import FastAPI, Header, HTTPException

from pydantic import BaseModel, Field

app = FastAPI(title="SIP alert starter")

API_KEY = os.environ["ALERT_API_KEY"]

BARESIP_CTRL = os.getenv("BARESIP_CTRL", "127.0.0.1:5555")

class CallRequest(BaseModel):

number: str = Field(min_length=2, max_length=64)

message: str = Field(default="Alarm", max_length=500)

def dial(number: str, message: str):

# Prefer a controlled Baresip command channel; confirm syntax for your build.

host, port = BARESIP_CTRL.split(":", 1)

try:

with socket.create_connection((host, int(port)), timeout=2) as s:

s.sendall((f"dial sip:{number}\n").encode())

return "ctrl_tcp"

except Exception:

# Fallback placeholder: adapt command, URI, audio file, and timeout.

cmd = ["baresip", "-f", "/etc/baresip", "-e", f"/dial sip:{number}"]

subprocess.Popen(cmd, start_new_session=True)

return "subprocess"

@app.post("/call")

def call(req: CallRequest, x_api_key: str = Header(default="")):

if x_api_key != API_KEY:

raise HTTPException(status_code=401, detail="invalid API key")

mode = dial(req.number, req.message)

return {"accepted": True, "mode": mode, "number": req.number}

## 7.3 Dockerfile sketch

FROM python:3.12-slim

# Install a pinned baresip package/build appropriate to your base image.

RUN apt-get update && apt-get install -y --no-install-recommends baresip ca-certificates \

&& rm -rf /var/lib/apt/lists/*

WORKDIR /app

COPY app.py /app/app.py

RUN pip install --no-cache-dir fastapi uvicorn pydantic

ENV ALERT_API_KEY=replace-at-runtime

EXPOSE 8080

CMD ["uvicorn", "app:app", "--host", "0.0.0.0", "--port", "8080"]

Mount a read-only Baresip config and an audio directory; pass secrets at runtime. Restrict ingress to ThingsBoard’s egress IPs or a private network/VPN. Do not expose port 8080 to the public internet.

# 8. ThingsBoard configuration

## 8.1 Rule chain

Start at the alarm/event path (or Message Type Switch → Alarm).

Add an alarm condition/filter that identifies the desired severity/device state.

Add a REST API Call node targeting the softphone service. Configure timeout and failure behavior appropriate to the node/version.

Add success and failure branches. On failure, create a visible ThingsBoard event/alarm, increment a failure metric, or route to a second notification path; do not silently discard the request.

Prevent duplicate calls with alarm state, deduplication, cooldown, or a queue policy.

URL: http://SOFTPHONE_HOST:8080/call

Header: X-API-Key: ${ALERT_API_KEY}

Body: {"number":"1002","message":"Alarm on ${deviceName}"}

Use HTTPS or a private network in any environment where the request crosses an untrusted network. Confirm the exact ThingsBoard REST node syntax for the deployed version; the template above is the intended payload shape.

## 8.2 Simulate telemetry

curl -v -X POST "http://THINGSBOARD_HOST/api/v1/ACCESS_TOKEN/telemetry" \

-H "Content-Type: application/json" \

-d '{"temperature":95}'

Use a test device and a rule threshold that is safe and reversible. Confirm the telemetry arrives, the alarm is created, and the REST node emits one request.

# 9. Test plan

## 9.1 Execution order

Verify the softphone registers and remains registered after a restart.

From the VM, issue a manual curl request to POST /call with the API key; confirm the softphone places a call.

Answer on the second Linphone endpoint and verify audio/message delivery.

Trigger a ThingsBoard alarm using simulated telemetry; confirm one REST request and one call.

Repeat with a verified PSTN target only after Phase 1 passes. Capture timestamps and logs for every attempt.

## 9.2 Test cases

| ID | Test / steps | Expected result | Pass / Fail | Notes |
| --- | --- | --- | --- | --- |
| TC-01 | Registration after restart: restart container, inspect SIP status. | Registered within agreed startup window; no manual repair. | ☐ Pass ☐ Fail |  |
| TC-02 | Registration renewal: run 24h; observe regint/keepalive. | Registration remains active; renewal logged. | ☐ Pass ☐ Fail |  |
| TC-03 | Alarm → call latency: trigger threshold and timestamp. | Target rings under ~5 seconds in normal test conditions. | ☐ Pass ☐ Fail |  |
| TC-04 | Answered-call audio: answer and listen to message. | Clear, intelligible two-way/alert audio as designed. | ☐ Pass ☐ Fail |  |
| TC-05 | No answer: let timeout expire. | Call ends cleanly; timeout and outcome are logged. | ☐ Pass ☐ Fail |  |
| TC-06 | Busy/reject: occupy or reject target. | Failure state is recorded and ThingsBoard branch is visible. | ☐ Pass ☐ Fail |  |
| TC-07 | Multiple simultaneous alarms: fire parallel test events. | Queue/concurrency policy behaves as designed; no lost events. | ☐ Pass ☐ Fail |  |
| TC-08 | Alarm clear: clear device condition. | Clear does not create an unintended duplicate call; state is logged. | ☐ Pass ☐ Fail |  |
| TC-09 | Provider unreachable: block provider or use invalid endpoint. | Failure is bounded, visible, and does not hang the REST node. | ☐ Pass ☐ Fail |  |
| TC-10 | DTMF acknowledgement (optional): press configured key. | DTMF is detected and acknowledgement posts back to ThingsBoard. | ☐ Pass ☐ Fail |  |
| TC-11 | NAT/audio: test VM and mobile on different networks. | RTP works; no one-way audio or unacceptable jitter. | ☐ Pass ☐ Fail |  |

## 9.3 Sign-off criteria

Alarm-to-ring latency is under approximately 5 seconds in the normal test environment (record actual samples).

100% of 50 test alarms produce a call or a documented, visible failure outcome with no silent loss.

The service remains registered for at least 24 hours, including a restart and registration renewal.

Failure branches, logs, and provider errors are visible to the team; credentials and billing controls are in place.

# 10. Alert message delivery options

Prerecorded audio: simplest deterministic path; normalize a short WAV and play it after answer.

TTS: generate audio with Piper or espeak ahead of the call or on demand; cache files and test pronunciation/latency.

DTMF acknowledgement (optional): collect a keypress, authenticate the action if needed, then POST an acknowledgement event/status back to ThingsBoard. Treat DTMF as advisory until retries, timeout, and spoofing behavior are tested.

# 11. Troubleshooting

| Symptom | Likely cause | Actions |
| --- | --- | --- |
| One-way / no audio | NAT/RTP path, firewall, or missing RTP range. | Open only the required RTP range, verify symmetric RTP/STUN/provider guidance, capture packet/log evidence, and retest across networks. |
| Registration drops | Keepalive, regint, NAT binding, clock drift. | Check NTP, reduce/adjust keepalive or regint per provider, inspect expiry/renewal logs, and restart-test. |
| Codec mismatch | Provider and client do not share a codec. | Start with PCMU/ulaw or PCMA/alaw as supported; align codec order and verify provider policy. |
| Headless audio errors | No sound device or incorrect driver. | Use null/aufile driver, a known-good WAV, and confirm container permissions/paths. |
| Trial limits | Only verified numbers, announcements, low credit, or restricted caller ID. | Verify the current trial rules; test your own verified number and monitor credit/usage. |
| Security exposure | Public endpoint, leaked API/SIP credentials. | Do not expose API publicly; require API key, IP allowlist/private networking, rotation, runtime secrets, and never commit credentials. |

# 12. Security & cost control

Use an API key at minimum; prefer HTTPS/private networking and allowlist only ThingsBoard egress addresses.

Store SIP passwords, API keys, and provider tokens in a secret manager or runtime environment; rotate after testing.

Apply rate limits, per-alarm cooldowns, maximum call duration, concurrency limits, and a provider spend cap where available.

Enable billing/usage alerts and review call records. Never assume a trial is unlimited or free; verify current terms and taxes.

Avoid buying numbers early. Use SIP-to-SIP free testing first, then trial PSTN to your own verified number, then procure production resources only after sign-off.

Do not use this test design for emergency calling without a separate compliance, location, resiliency, and provider review.

# 13. Test results log template

| Run | Date/time | Phase / target | Alarm ID | Ring latency | Answered/audio | Outcome / error | Tester |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |

# 14. Sign-off

| Field | Value |
| --- | --- |
| Tester / owner |  |
| Date |  |
| Environment (TB host, VM, provider, target) |  |
| Documented test count / pass rate |  |
| Result (Pass / Conditional / Fail) |  |
| Open issues / follow-up owner |  |

Approvals / notes:

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

End of guide